PROJECT TITLE: PIN BASED ALARM SYSTEM

| NAME | ROLL NO. | SECTION | ROLE |
| --- | --- | --- | --- |
| ANSER GHAFFAR | SU92-BSDSM-F23-032 | BSDS-6A | LOGIC IMPLEMENTATION AND DOCUMENTATION |
| ABDUL HANAN | SU92-BSDSM-F23-021 | BSDS-6A | VALIDATION |

Github repo link: https://github.com/AnserGhaffar/COAL_PROJECT

PIN-Based Security Alarm System

This is a beginner-level 8086 Assembly Language mini project for DOSBox. The system asks the user to enter a 4-digit numeric PIN. After three wrong attempts, the system enters lockdown mode and shows a blinking warning message until reset.

Project Objectives

Learn keyboard input using INT 21h

Understand loops and counters

Use conditional jumps for decision making

Implement a simple security system

Learn screen handling using INT 10h

Main Features

4-digit PIN authentication

Only numeric digits are accepted

Maximum 3 attempts allowed

Access Granted and Access Denied messages

Lockdown mode after 3 failed attempts

Blinking security warning message

Reset system using R key

Interrupts Used

| Interrupt | Function | Purpose |
| --- | --- | --- |
| INT 21h | AH=09h | Display string |
| INT 21h | AH=02h | Display character |
| INT 21h | AH=07h | Read keyboard input |
| INT 21h | AH=0Bh | Check keyboard status |
| INT 10h | AH=06h | Clear screen |
| INT 10h | AH=02h | Move cursor |

Registers Used

AX – used for interrupts and calculations

BX – used as delay counter

CX – used for loops

DX – used for displaying messages

SI – points to entered PIN

DI – points to correct PIN

Working of the Program

Program starts and clears the screen

User enters a 4-digit PIN

Program accepts only digits 0-9

Entered PIN is compared with stored PIN

If correct, Access Granted message is displayed

If incorrect, attempts counter increases

After 3 wrong attempts, lockdown mode starts

Blinking warning message appears

Pressing R resets the system

Compilation and Running Steps

1. Save the code as project.asm
2. Open DOSBox
3. Mount folder:
   mount c c:\asm
4. Change drive:
   c:
5. Compile:
   nasm -f bin coal_project.asm -o pin.com
6. Run:
   project

Conclusion

This project demonstrates basic 8086 Assembly Language concepts including interrupts, loops, arrays, counters, keyboard handling, conditional jumps, and screen manipulation in a simple and interactive way.

Code and Output